Replacing CAMs

Make sure the letters cast into the end of the gear mount at the center of the cam are up.

Align the lines marked on the sprocket with the top face of the head.

Must do the rear cylinders (number 1 and 3) first.

After done with the rear, you can do the front cylinders (number 2 and 4).

Next step is a Trick Step, here is where I made mistake my first time and had to do over:

Rotate the crank 90 degrees ( 1/4 turn ) COUNTER CLOCKWISE to the T2-4 mark. Now align the index DOTS with the top face of the head.

When done, and at T1-2, and rear cam lettering is on top, and rear cams have the sprocket lines aligned with the head face, the front cylinders cams will also lign up with the lines aligning with the front head face.

(1) When your cams are timed properly, ie all cam gear lines lined up with the top of the engine at T 1-3 mark, either all the text on your cams (ER,IR,IF,EF) should be facing up at the same time, or all will be down at the same time.

- or –

(2) Rotate the engine (clockwise) using the alternator bolt (left side of engine) until the T 1-3 mark on the alternator lines up with the rear case split line. Your cams should all have a line marked across the cam gear that you can see from the left side of the engine- ALL 4 gears should line up with the top surface of the head. Rotate the engine clockwise until the T 2-4 mark lines up with the case split line. ALL 4 cam gears should have DOTS lined up with the top surface of the head.